# Adaptive Workflow POC - Comprehensive Report

# Final Summary & Recommendations

This document consolidates all details from the initial setup, architecture, automation steps, issues resolved, and lessons learned during the Adaptive Workflow POC. It includes architecture diagrams, Lambda code updates, Step Function configurations, and VPC networking fixes (NAT Gateway setup). The automation now ensures real-time triggering of workflows based on RDS data changes, with proper error handling and monitoring recommendations.

# Adaptive Workflow Automation POC Report

# Planned Objectives vs Achieved Outcomes

| Planned Objectives | Achieved Outcomes |
| --- | --- |
| Trigger Step Function on new RDS record | Automated trigger via EventBridge + Lambda |
| Handle machine states dynamically | Step Function executes based on MachineStatus |
| Ensure VPC connectivity | NAT Gateway added for outbound internet |
| Avoid duplicate triggers | ProcessedFlag implemented in DB |

# Key Achievements

End-to-end automation from RDS to Step Functions

Correct JSON input structure for workflow execution

Resolved VPC connectivity issues using NAT Gateway

Enhanced Lambda with logging and error handling

# Lessons Learned

Always validate IAM permissions for Lambda and EventBridge

Ensure Lambda timeout accommodates DB + API calls

VPC configuration is critical for outbound AWS service calls

Use Step Functions Parameters to simplify Lambda input

# Future Roadmap

Move from polling to real-time streaming using AWS DMS + EventBridge

Add monitoring and alerts via CloudWatch and SNS

Implement retry and error handling in Step Functions

Integrate approval workflow with external systems securely

# Adaptive Workflow POC Summary

# Overview

This document summarizes the Adaptive Workflow Proof of Concept (POC) implementation, including architecture, automation steps, issues encountered, fixes applied, and lessons learned.

# Initial Setup

The POC started with a Step Function designed to handle machine states: Running → Success, Down → Reroute, Maintenance → Maintenance Workflow. Manual testing confirmed correct execution.

# Automation Achieved

EventBridge Scheduler triggers Lambda every minute. Lambda connects to SQL Server RDS, fetches unprocessed records, triggers Step Function with correct JSON input, and updates ProcessedFlag. NAT Gateway was added to enable Lambda's outbound internet access for Step Functions API calls.

# Key Issues & Fixes

1. Lambda timeout errors resolved by increasing timeout and adding NAT Gateway.
2. IAM permissions updated for states:StartExecution.
3. Corrected Step Function input structure to match workflow requirements.
4. Adjusted JSONPath mappings in nested workflows to avoid runtime errors.

# Lessons Learned

• Always validate VPC networking for Lambda when calling AWS public services.
• Use ProcessedFlag to prevent duplicate triggers.
• Ensure consistent naming conventions in Step Functions and Lambda payloads.
• Add detailed logging for easier troubleshooting.

# Scaling Recommendations

For production: Replace polling with RDS → DMS → EventBridge streaming for real-time triggers. Add CloudWatch alarms and SNS notifications for monitoring. Optimize Lambda concurrency and Step Function retries for high-volume scenarios.

| Component | Description |
| --- | --- |
| AWS Step Functions | Orchestrates machine state workflows |
| AWS Lambda | Fetches data from RDS and triggers Step Functions |
| Amazon RDS (SQL Server) | Stores MESData records |
| Amazon EventBridge | Schedules Lambda for polling |
| NAT Gateway | Provides outbound internet for Lambda in VPC |