Hybrid MES in Practice: Real-Time Edge-to-Cloud Communication

1. Architecture Overview

This use case demonstrates a hybrid MES architecture integrating edge-level OPC UA data acquisition with cloud-based data storage and analytics. The system enables real-time communication between an OPC UA server running on a local edge device and an AWS-hosted cloud infrastructure using MQTT protocol.

Flow Diagram (Textual):

[OPC UA Server (Edge)] → [Python MQTT Publisher] → [RabbitMQ MQTT Broker (AWS EC2)] → [Python MQTT Subscriber] → [AWS RDS (SQL Server)]

2. Technologies Used

- OPC UA Server & Client: Simulated using Python and Matrikon SDK.

- MQTT Protocol: Lightweight messaging protocol for real-time data exchange.

- RabbitMQ MQTT Plugin: Hosted on AWS EC2 for MQTT message brokering.

- AWS EC2: Hosts RabbitMQ and MQTT subscriber.

- AWS RDS (SQL Server): Stores real-time production/quality data.

- Python Libraries:

  - opcua

  - paho-mqtt

  - pyodbc

  - mysql-connector-python

3. Implementation Steps

Edge Setup:

- Installed OPC UA server and client locally.

- Generated random values using Python and C++ scripts.

- Identified correct NodeIds using UaExpert and Python browsing script.

MQTT Publisher:

- Subscribed to OPC UA variables.

- Published combined JSON payload to RabbitMQ MQTT broker.

Cloud Setup:

- Configured RabbitMQ on AWS EC2.

- Enabled persistent connections and authentication.

- Created SQL Server RDS instance and schema.

MQTT Subscriber:

- Subscribed to MQTT topic.

- Parsed JSON payload.

- Inserted data into RDS with timestamp conversion.

4. Key Python Scripts

from opcua import Client, ua

import random, time

client = Client("opc.tcp://localhost:55000")

client.connect()

node = client.get_node("ns=1;s=Objects/MyFolder/MyStoredVariable")

while True:

value = random.randint(0, 100)

node.set_value(ua.Variant(value, ua.VariantType.Int32))

time.sleep(1)

✅ MQTT Publisher (OPC UA → MQTT):

from opcua import Client

import paho.mqtt.client as mqtt

import json, time

# Subscribe to multiple variables and publish combined JSON

✅ MQTT Subscriber (MQTT → RDS):

from datetime import datetime

import pyodbc, paho.mqtt.client as mqtt, json

# Parse timestamp with microseconds

parsed_timestamp = datetime.strptime(raw_timestamp.strip(), "%Y-%m-%d %H:%M:%S.%f")

5. Troubleshooting Notes

- BadTypeMismatch: Resolved by matching OPC UA variable data types (e.g., Int32, UInt32).

- NodeId Not Found: Fixed using UaExpert and Python browsing script.

- Timestamp Conversion Error: Solved by parsing ISO format with microseconds.

- None Values in MQTT: Caused by inactive OPC UA updates; resolved by running generator script in parallel.

6. Final Working Flow

- OPC UA server updates variables with random values.

- MQTT publisher detects changes and sends JSON payload to RabbitMQ.

- MQTT subscriber receives messages and inserts them into AWS RDS.

- Data is stored with accurate timestamps and ready for analytics.

{

"timestamp": "2025-11-03 06:45:23.966780",

"variables": {

"MyStoredVariable": 29,

"MySynchronousVariable": 10,

"MyAsynchronousVariable": 210

}

}